
ATS-900-V-15-15 COMPONENT SIZING 机组及主要部件
WINDOW:



30 cm X 30 cm (12” W X 12” H) (optional)
观察窗：                                                    30 cm X30 cm（可选）
COMPRESSOR SIZE:


 (2) 15 Hp (water-cooled)

压缩机：                                                     两个15 马力（水冷）
HEATER:



Approximately 27 kW for hot zone (compartment)





Approximately   6 kW for cold/hot zone (compartment)

加热器：                                                     约27千瓦，用于热箱
约6千瓦，用于冷/热箱

CIRCULATOR MOTORS:
               (2) 3 Hp, one per compartment

循环风机：                                                  两个3马力，每箱各一个
INTERNAL AIRFLOW:

Approximately 96 cubic meters/minute (3400 CFM)  rated in cold compartment






Approximately 130 cubic meters/minute (4600 CFM)  rated in cold compartment

风量：                                                         冷箱约3400 CFM
                                                                     热箱约4600 CFM
ATS-900-V-15-15 PERFORMANCE 性能
TEMPERATURE RANGE:

HOT ZONE:


 140°F to 392°F
(60°C to 200°C)


COLD ZONE:


-100°F to 350°F
(-73°C to 177°C)
 

温度范围：        

热箱：                                        60°C 至 200°C
冷箱：                                       -73°C 至 177°C
TEST TEMPERATURE RANGE*

Maximum Load**

with LN2  Optional Boost:

-55°C to   85°C (-67°F to 185°F)
                              54 kg (120 lbs)

                    54 kg (120 lbs)




-65°C to 150°C (-85°F to 302°F)
                              18 kg (40 lbs)

                    54 kg (120 lbs)

-65°C to 200°C (-85°F to 392°F)
                              11 kg (25 lbs)

                    54 kg (120 lbs)
测试温度范围*                                                             最大负载**                                   可选的液氮辅助：

-55°C 至 85°C
                                                             54 公斤

                        54  公斤
-65°C 至 150°C
                                                             18公斤


          54  公斤

-65°C至 200°C                                                               11 公斤                                             54  公斤
*Performance is based on a 5 minute recovery in the return air stream and 4°C temperature offsets at the extremes

*性能基于在回风口处测量，恢复时间为5分钟，恢复时间是基于当温度补偿设定为4℃时

** Product load consists of 12 gauge steel plates and the product carrier. 
**产品负载由12 gauge厚的钢板和产品提篮组成 

The weight of any additional shelves (7 lbs each) must be included as part of the test loads shown above

任何附加的试件架(每个7磅)都必须作为以上所述测试负载的一部分

PAGE  
2

